CN7000 Solution and Feature Matrices

Requirements

Rev. Draft

TBD

Table of Contents

1	Scale-Up Solutions	1

2	Feature Coexistence Rules	1

2.1	Scale-Out Mandatory Combinations:	1

2.2	Scale-Up Mandatory Combinations:	1

2.3	Mutually Exclusive Configurations:	1

3	Feature Applicability by Solution	1

4	Key Differentiators by Solution Type	1

5	Glossary	1

Table of Tables

Table 1-1: Scale-Out AI Workload Solutions	4

Table 1-2: Scale-Out HPC Workload Solutions	5

Table 1-1: Scale-Up AI Workload Solutions	7

Table 1-2: Scale-Up HPC Workload Solutions	8

Table 3-1: Feature Applicability by Solution	11

Table 4-1: Key Differentiators by Solution Type	13

Table 1-1: Scale-Out AI Workload Solutions

| Feature Category | Sub-Feature | Small-Medium Scale | Large Scale | Hyperscalar |
| --- | --- | --- | --- | --- |
| Protocols | Primary | UE+, UE, Ethernet | UE+, UE, Ethernet | UE+, UE, Ethernet |
|  | Secondary | RoCE (interop) | RoCE (interop) | RoCE (interop) |
|  | Dynamic Select | ✓ (UE+, UE, Ethernet, RoCE) | ✓ (UE+, UE, Ethernet, RoCE) | ✓ (UE+, UE, Ethernet, RoCE) |
| Transports (UE) | PDS Modes | RUD, ROD, RUDI, UUD | RUD, ROD, RUDI, UUD | RUD, ROD, RUDI, UUD |
|  | SES | Tagged Send, RMA, Atomic, Deferrable Send | Tagged Send, RMA, Atomic, Deferrable Send | Tagged Send, RMA, Atomic, Deferrable Send |
|  | CMS | NSCC, RCCC | NSCC, RCCC | NSCC, RCCC |
| Scale | Max Endpoints | ≤10K (≤1.25K/rail) | 10K-93K (1.25K-11.6K/rail) | 100K-1.44M (12.5K-180K/rail) |
|  | Topology | Fat-Tree or Fat-Tree Rail | Fat-Tree Rail | Fat-Tree Rail |
| Interoperability | Mode | Homogeneous | Homogeneous or Heterogeneous (island) | Heterogeneous (island) |
|  | Ethernet Compat | UE standard compliant | UE standard compliant | UE standard compliant |
| Value-Add: Performance | Link Behavior | Lossless (CBFC, LLR) | Lossless (CBFC, LLR) | Lossless (CBFC, LLR) |
|  | Message Rate | 4B msgs/sec bidi | 4B msgs/sec bidi | 4B msgs/sec bidi |
|  | Latency (hop) | ≤250ns | ≤250ns | ≤250ns |
| Value-Add: Routing | SDR/Packet Spray | ✓ | ✓ | ✓ |
|  | FGAR | ✓ | ✓ | ✓ |
|  | ECAR | ✓ | ✓ | ✓ |
|  | Dynamic Route Recovery | ✓ | ✓ | ✓ |
| Value-Add: Congestion Mgmt | ECN | ✓ (UE, Ethernet, RoCE) | ✓ (UE, Ethernet, RoCE) | ✓ (UE, Ethernet, RoCE) |
|  | NSCC | ✓ (lossless, lossy) | ✓ (lossless, lossy) | ✓ (lossless, lossy) |
|  | RCCC | ✓ | ✓ | ✓ |
|  | CSIG/CSIG+ | ✓ | ✓ | ✓ |
| Value-Add: Collective Accel | In-Network | ✓ (homogeneous/island) | ✓ (homogeneous/island) | ✓ (homogeneous/island) |

Table 1-2: Scale-Out HPC Workload Solutions

| Feature Category | Sub-Feature | Small Scale | Medium Scale | Large Scale |
| --- | --- | --- | --- | --- |
| Protocols | Primary | OPA 16B, UE+, UE | OPA 16B, UE+, UE | OPA 16B, UE+, UE |
|  | Secondary | Ethernet, RoCE | Ethernet, RoCE | Ethernet, RoCE |
|  | Dynamic Select | ✓ (OPA, UE+, UE, Ethernet, RoCE) | ✓ (OPA, UE+, UE, Ethernet, RoCE) | ✓ (OPA, UE+, UE, Ethernet, RoCE) |
| Transports (UE) | PDS Modes | RUD, ROD, RUDI, UUD | RUD, ROD, RUDI, UUD | RUD, ROD, RUDI, UUD |
|  | SES | Tagged Send, RMA, Atomic, Deferrable Send | Tagged Send, RMA, Atomic, Deferrable Send | Tagged Send, RMA, Atomic, Deferrable Send |
|  | CMS | NSCC, RCCC | NSCC, RCCC | NSCC, RCCC |
| Scale | Max Endpoints | ≤5K (≤625/rail) | 5K-10K (625-1.25K/rail) | 10K-93K (1.25K-11.6K/rail) |
|  | Topology | Fat-Tree or Fat-Tree Rail | Fat-Tree or Fat-Tree Rail | Fat-Tree Rail |
| Interoperability | Mode | Homogeneous | Homogeneous | Homogeneous or Heterogeneous (island) |
|  | Ethernet Compat | UE standard compliant | UE standard compliant | UE standard compliant |
|  | CN5000/CN6000 | No | No | No |
| Value-Add: Performance | Link Behavior | Lossless (CBFC, LLR) | Lossless (CBFC, LLR) | Lossless (CBFC, LLR) |
|  | Message Rate | 4B msgs/sec bidi | 4B msgs/sec bidi | 4B msgs/sec bidi |
|  | Latency (hop) | ≤250ns | ≤250ns | ≤250ns |
| Value-Add: Routing | SDR/Packet Spray | ✓ | ✓ | ✓ |
|  | FGAR | ✓ | ✓ | ✓ |
|  | ECAR | ✓ | ✓ | ✓ |
|  | Dynamic Route Recovery | ✓ | ✓ | ✓ |
| Value-Add: Congestion Mgmt | ECN | ✓ (UE, Ethernet, RoCE) | ✓ (UE, Ethernet, RoCE) | ✓ (UE, Ethernet, RoCE) |
|  | NSCC | ✓ (lossless, lossy) | ✓ (lossless, lossy) | ✓ (lossless, lossy) |
|  | RCCC | ✓ | ✓ | ✓ |
|  | CSIG/CSIG+ | ✓ | ✓ | ✓ |
| Value-Add: Collective Accel | In-Network | ✓ (homogeneous/island) | ✓ (homogeneous/island) | ✓ (homogeneous/island) |
| Value-Add: Advanced Topo | Megafly | Optional | Optional | ✓ |
|  | Dragonfly | ✓ | ✓ | ✓ |

# Scale-Up Solutions

Table 1-1: Scale-Up AI Workload Solutions

| Feature Category | Sub-Feature | Pod/Rack Scale | Multi-Rack Scale |
| --- | --- | --- | --- |
| Protocols | Primary | UALink 200G | UALink 200G |
|  | Secondary | UE (ULN variant) | UE (ULN variant) |
|  | Dynamic Select | N/A | N/A |
| Transports (UALink) | Semantics | Memory (load/store) | Memory (load/store) |
|  | DL/TL | Fixed 680B payload, VC, LLR, CBFC | Fixed 680B payload, VC, LLR, CBFC |
|  | Ordering | Same-address ordering | Same-address ordering |
| Transports (UE/ULN) | PDS Modes | ROD, RUD | ROD, RUD |
|  | CMS | RCCC | RCCC |
| Scale | Max Endpoints | ≤1K accelerators | ≤1K accelerators |
|  | Topology | Single-tier (1-hop) | Single-tier (1-4 racks) |
|  | Cable Length | <4m per link | <4m per link |
|  | RTT Target | <1μs | <1μs |
| Interoperability | Mode | Homogeneous | Homogeneous |
|  | Standards | UALink Consortium spec | UALink Consortium spec |
| Value-Add: Performance | Link Behavior | Lossless (CBFC or PFC) | Lossless (CBFC or PFC) |
|  | Message Rate | 4B msgs/sec bidi | 4B msgs/sec bidi |
|  | Latency (RTT) | Sub-μs target | Sub-μs target |
|  | Bandwidth/XPU | 1.6T (8x200G lanes) | 1.6T (8x200G lanes) |
| Value-Add: Routing | SDR/Packet Spray | N/A | N/A |
|  | FGAR | N/A | N/A |
|  | ECAR | N/A | N/A |
|  | Dynamic Route Recovery | N/A | N/A |
| Value-Add: Congestion Mgmt | ECN | N/A | N/A |
|  | NSCC | N/A | N/A |
|  | RCCC | ✓ (UE/ULN) | ✓ (UE/ULN) |
|  | CSIG/CSIG+ | N/A | N/A |
|  | Flow Control | PFC or CBFC | PFC or CBFC |
| Value-Add: Collective Accel | In-Network | Optional | Optional |
| Value-Add: Advanced Topo | Megafly | No | No |
|  | Dragonfly | No | No |

Note: Cornelis does not currently have a scale-up NIC or embedded accelerator HFI. Scale-up solutions assume different endpoint vendors connecting to Cornelis switches.

Table 1-2: Scale-Up HPC Workload Solutions

| Feature Category | Sub-Feature | Pod/Rack Scale | Multi-Rack Scale |
| --- | --- | --- | --- |
| Protocols | Primary | UE (ULN) | UE (ULN) |
|  | Secondary | OPA (homogeneous) | OPA (homogeneous) |
|  | Dynamic Select | ✓ (OPA, UE, Ethernet, RoCE) | ✓ (OPA, UE, Ethernet, RoCE) |
| Transports (UE/ULN) | PDS Modes | ROD, RUD | ROD, RUD |
|  | SES | RMA, Tagged Send | RMA, Tagged Send |
|  | CMS | RCCC, NSCC | RCCC, NSCC |
| Scale | Max Endpoints | ≤1K compute nodes | ≤1K compute nodes |
|  | Topology | Single-tier, all-to-all | Single-tier, 1-4 racks |
|  | Cable Length | <4m | <4m |
|  | RTT Target | <1μs | <1μs |
| Interoperability | Mode | Homogeneous | Homogeneous |
|  | Standards | UE HPC profile | UE HPC profile |
| Value-Add: Performance | Link Behavior | Lossless (CBFC, LLR) | Lossless (CBFC, LLR) |
|  | Message Rate | 4B msgs/sec bidi | 4B msgs/sec bidi |
|  | Latency (RTT) | Sub-μs target | Sub-μs target |
|  | Bandwidth/Node | 1.6T (aggregated) | 1.6T (aggregated) |
| Value-Add: Routing | SDR/Packet Spray | N/A | N/A |
|  | FGAR | N/A | N/A |
|  | ECAR | N/A | N/A |
|  | Dynamic Route Recovery | N/A | N/A |
| Value-Add: Congestion Mgmt | ECN | ✓ | ✓ |
|  | NSCC | ✓ (lossless, lossy) | ✓ (lossless, lossy) |
|  | RCCC | ✓ | ✓ |
|  | CSIG/CSIG+ | Optional | Optional |
| Value-Add: Collective Accel | In-Network | ✓ (homogeneous/island) | ✓ (homogeneous/island) |
| Value-Add: Advanced Topo | Megafly | No | No |
|  | Dragonfly | No | No |

Note: Cornelis does not currently have a scale-up NIC or embedded accelerator HFI. Scale-up solutions assume different endpoint vendors connecting to Cornelis switches.

# Feature Coexistence Rules

## Scale-Out Mandatory Combinations:

- UE+ Protocol → Requires: CBFC + LLR + (NSCC or RCCC) + CSIG capability

- Large Scale Deployments → Requires: FGAR + CSIG+ + Fat-Tree Rail topology

- Heterogeneous (island) Interop → Requires: Standard UE (not UE+) + UE-compliant congestion control

- Dynamic Protocol Selection → Requires: Homogeneous Cornelis deployment

## Scale-Up Mandatory Combinations:

- UALink (Scale-Up AI) → Requires: CBFC (or PFC) + LLR + Single-tier topology + Memory semantics

- UE/ULN (Scale-Up HPC) → Requires: RCCC + Sub-μs RTT + CBFC + Single-tier topology

- Collective Acceleration → Requires: Homogeneous/island deployment

## Mutually Exclusive Configurations:

- Scale-Out ⊕ Scale-Up on same switch instance (different deployment models)

- Dynamic protocol selection ⊕ Heterogeneous (island) (dynamic selection only for homogeneous Cornelis)

- Advanced Topologies (Megafly/Dragonfly) ⊕ Scale-Up (Scale-Up uses single-tier only)

- Advanced Topologies (Megafly/Dragonfly) ⊕ Scale-Out AI (HPC only)

# Feature Applicability by Solution

Table 3-1: Feature Applicability by Solution

| Feature Category | Sub-Feature | Scale-Out AI | Scale-Out HPC | Scale-Up AI | Scale-Up HPC |
| --- | --- | --- | --- | --- | --- |
| Protocols | UE+, UE, Ethernet | ✓ | ✓ | — | ✓ (ULN) |
|  | OPA 16B | — | ✓ | — | ✓ |
|  | UALink | — | — | ✓ | — |
|  | RoCE | ✓ (interop) | ✓ (interop) | — | ✓ (interop) |
|  | Dynamic Select | ✓ | ✓ | — | ✓ |
| Transports (UE) | PDS Modes | ✓ | ✓ | ✓ (subset) | ✓ (subset) |
|  | SES | ✓ | ✓ | — | ✓ (subset) |
|  | CMS | ✓ | ✓ | ✓ (RCCC) | ✓ |
| Transports (UALink) | Memory Semantics | — | — | ✓ | — |
| Scale | >10K Endpoints | ✓ | ✓ | — | — |
|  | ≤1K Endpoints | ✓ | ✓ | ✓ | ✓ |
|  | Multi-tier Topology | ✓ | ✓ | — | — |
|  | Single-tier Topology | ✓ | ✓ | ✓ | ✓ |
| Interoperability | Homogeneous | ✓ | ✓ | ✓ | ✓ |
|  | Heterogeneous (island) | ✓ (Large+) | ✓ (Large) | — | — |
| Value-Add: Performance | Lossless (CBFC, LLR) | ✓ | ✓ | ✓ | ✓ |
|  | 4B msgs/sec bidi | ✓ | ✓ | ✓ | ✓ |
|  | ≤250ns hop latency | ✓ | ✓ | — | — |
|  | Sub-μs RTT | — | — | ✓ | ✓ |
| Value-Add: Routing | SDR/Packet Spray | ✓ | ✓ | — | — |
|  | FGAR | ✓ | ✓ | — | — |
|  | ECAR | ✓ | ✓ | — | — |
|  | Dynamic Route Recovery | ✓ | ✓ | — | — |
| Value-Add: Congestion Mgmt | ECN | ✓ | ✓ | — | ✓ |
|  | NSCC | ✓ | ✓ | — | ✓ |
|  | RCCC | ✓ | ✓ | ✓ | ✓ |
|  | CSIG/CSIG+ | ✓ | ✓ | — | Optional |
| Value-Add: Collective Accel | In-Network | ✓ | ✓ | Optional | ✓ |
| Value-Add: Advanced Topo | Megafly | — | ✓ | — | — |
|  | Dragonfly | — | ✓ | — | — |

# Key Differentiators by Solution Type

Table 4-1: Key Differentiators by Solution Type

| Dimension | Scale-Out AI | Scale-Out HPC | Scale-Up AI | Scale-Up HPC |
| --- | --- | --- | --- | --- |
| Primary Goal | Maximize GPU utilization, collective performance | Balanced latency/bandwidth/message rate | Ultra-low latency, memory semantics | Low-latency RDMA within pod |
| Primary Protocol | UE+, UE, Ethernet | OPA 16B, UE+, UE | UALink | UE (ULN) |
| Packet Size | Variable MTU | Variable MTU | Fixed 680B (UALink) | Variable MTU |
| Max Scale | 1.44M endpoints | 93K endpoints | ≤1K accelerators | ≤1K compute nodes |
| Topology | Fat-Tree, Fat-Tree Rail | Fat-Tree, Fat-Tree Rail, Megafly, Dragonfly | Single-tier | Single-tier |
| Routing Complexity | High (multi-tier, adaptive) | High (multi-tier, adaptive) | None (single-tier) | None (single-tier) |
| Congestion Strategy | NSCC + RCCC | NSCC + RCCC | RCCC (UE/ULN) | NSCC + RCCC |
| Telemetry | CSIG/CSIG+ | CSIG/CSIG+ | N/A | Optional |
| Collective Offload | ✓ (homogeneous/island) | ✓ (homogeneous/island) | Optional | ✓ (homogeneous/island) |
| Interoperability | Homogeneous or Heterogeneous (island) | Homogeneous or Heterogeneous (island) | Homogeneous | Homogeneous |

References: - CN7000 Switch Requirements (Landing Zone) - CN7000 HFI Requirements (Landing Zone)
- UEC Transport Specifications (PDS, SES, CMS) - UALink Consortium Specifications - CN7000 Features Documentation

# Glossary

bidi - Bidirectional. Indicates that a metric (e.g., message rate) applies to both transmit and receive directions combined.

CBFC - Credit-Based Flow Control. A lossless flow control mechanism that uses credits to manage buffer space and prevent packet drops.

CMS - Congestion Management Service. UE transport layer service for congestion detection and response.

CSIG/CSIG+ - Congestion Signaling. Telemetry mechanism for communicating congestion state (2B/4B/6B variants).

ECAR - Entropy Controlled Adaptive Routing. Routing algorithm that uses entropy-based path selection to balance load.

ECN - Explicit Congestion Notification. Standard mechanism for signaling network congestion to endpoints.

FGAR - Fine-Grained Adaptive Routing. Telemetry-based adaptive routing for optimal path selection.

LLR - Link-Level Retry. Mechanism for retransmitting corrupted or lost packets at the link layer.

NSCC - Network-Side Congestion Control. Congestion control mechanism supporting both lossless and lossy modes.

OPA - Omni-Path Architecture. Cornelis proprietary high-performance fabric protocol.

PDS - Packet Delivery Service. UE transport layer service defining delivery semantics (RUD, ROD, RUDI, UUD).

RCCC - Receiver Credit-based Congestion Control. Congestion control using receiver-side credit management.

RMA - Remote Memory Access. One-sided communication operations for direct memory access.

ROD - Reliable Ordered Delivery. PDS mode guaranteeing in-order packet delivery.

RoCE - RDMA over Converged Ethernet. Standard for RDMA operations over Ethernet networks.

RUD - Reliable Unordered Delivery. PDS mode guaranteeing delivery without ordering constraints.

RUDI - Reliable Unordered Delivery with Immediate data. RUD variant with immediate data support.

SDR - Source-based Deterministic Routing. Static routing determined at the source endpoint.

SES - Send/Event Service. UE transport layer service for messaging semantics (Tagged Send, RMA, Atomic).

UALink - Ultra Accelerator Link. Consortium-defined standard for accelerator interconnect with memory semantics.

UE - Ultra Ethernet. Industry-standard high-performance Ethernet protocol for HPC/AI workloads.

UE+ - Ultra Ethernet Plus. Cornelis-enhanced UE with additional performance features.

ULN - Ultra Local Network. UE variant optimized for scale-up, low-latency deployments.

UUD - Unreliable Unordered Delivery. PDS mode with best-effort delivery semantics.