Table of Contents

List of Tables

Now let me create the comprehensive matrices for CN7000 solutions. Based on the documentation, I'll organize this as detailed tables showing Scale-Out and Scale-Up solutions.

# CN7000 Solution and Feature Matrices

## SCALE-OUT SOLUTIONS

### Table 1: Scale-Out AI Workload Solutions

| Feature Category | Protocol | CN7000 Small-Medium Scale | CN7000 Large Scale | CN7000 Hyperscalar |
| --- | --- | --- | --- | --- |
| Protocols | Primary | UE+, UE, Ethernet | UE+, UE, Ethernet | UE+, UE, Ethernet |
|  | Secondary | RoCE (interop) | RoCE (interop) | RoCE (interop) |
|  | OPA | OPA 16B (homogeneous) | OPA 16B (homogeneous) | OPA 16B (homogeneous) |
| Transports (UE) | PDS Modes | RUD, ROD, RUDI, UUD | RUD, ROD, RUDI, UUD | RUD, ROD, RUDI, UUD |
|  | SES | Tagged Send, RMA, Atomic, Deferrable Send | Tagged Send, RMA, Atomic, Deferrable Send | Tagged Send, RMA, Atomic, Deferrable Send |
|  | CMS | NSCC, RCCC | NSCC, RCCC | NSCC, RCCC |
| Scale | Max Endpoints | 14K-27K | 83K-260K | 1M+ |
|  | Topology | 2-3 tier Fat Tree | 3-4 tier Fat Tree, Dragonfly | 4+ tier Fat Tree, Dragonfly, Megafly |
| Interoperability | Mode | Homogeneous (Cornelis-only) | Homogeneous or Heterogeneous | Heterogeneous (multi-vendor) |
|  | Ethernet Compat | UE standard compliant | UE standard compliant | UE standard compliant |
| Value-Add: Performance | Link Behavior | Lossless (CBFC, LLR) | Lossless (CBFC, LLR) | Lossless (CBFC, LLR) |
|  | Message Rate | 4B msgs/sec bidirectional | 4B msgs/sec bidirectional | 4B msgs/sec bidirectional |
|  | Latency (hop) | ~110ns switch hop | ~110ns switch hop | ~110ns switch hop |
| Value-Add: Routing | SDR/Packet Spray | ✓ (static + dynamic spray) | ✓ (static + dynamic spray) | ✓ (static + dynamic spray) |
|  | FGAR | ✓ (Fine-Grained Adaptive) | ✓ (Fine-Grained Adaptive) | ✓ (Fine-Grained Adaptive) |
|  | ECAR | ✓ (Enhanced Congestion) | ✓ (Enhanced Congestion) | ✓ (Enhanced Congestion) |
|  | Fast Route Recovery | ✓ (dynamic reroute <3μs) | ✓ (dynamic reroute <3μs) | ✓ (dynamic reroute <3μs) |
| Value-Add: Congestion Mgmt | ECN | ✓ (UE, Ethernet) | ✓ (UE, Ethernet) | ✓ (UE, Ethernet) |
|  | NSCC | ✓ (RTT-based, lossy) | ✓ (RTT-based, lossy) | ✓ (RTT-based, lossy) |
|  | RCCC | ✓ (receiver credit control) | ✓ (receiver credit control) | ✓ (receiver credit control) |
|  | CSIG/CSIG+ | ✓ (2B/4B/6B telemetry) | ✓ (2B/4B/6B telemetry) | ✓ (2B/4B/6B telemetry) |
| Value-Add: Collective Accel | In-Network | ✓ (RISC-V line-rate reduce) | ✓ (RISC-V line-rate reduce) | ✓ (RISC-V line-rate reduce) |
| Value-Add: Advanced Topo | Megafly | Optional | ✓ Supported | ✓ Optimized |
|  | Dragonfly | ✓ Supported | ✓ Supported | ✓ Supported |

### Table 2: Scale-Out HPC Workload Solutions

| Feature Category | Protocol | CN7000 Small-Medium Scale | CN7000 Large Scale | CN7000 Hyperscalar |
| --- | --- | --- | --- | --- |
| Protocols | Primary | OPA 16B, UE+, UE | OPA 16B, UE+, UE | OPA 16B, UE+, UE |
|  | Secondary | Ethernet, RoCE | Ethernet, RoCE | Ethernet, RoCE |
|  | Dynamic Select | ✓ (OPA+UE+Eth per-packet) | ✓ (OPA+UE+Eth per-packet) | ✓ (OPA+UE+Eth per-packet) |
| Transports (UE) | PDS Modes | RUD, ROD, UUD | RUD, ROD, UUD | RUD, ROD, UUD |
|  | SES | Tagged Send, RMA, Atomic | Tagged Send, RMA, Atomic | Tagged Send, RMA, Atomic |
|  | CMS | NSCC, RCCC | NSCC, RCCC | NSCC, RCCC |
| Transports (OPA) | Mode | 16B packet format | 16B packet format | 16B packet format |
|  | Reliability | LLR + Credit-based FC | LLR + Credit-based FC | LLR + Credit-based FC |
| Scale | Max Endpoints | 14K-27K | 83K-260K | 1M+ |
|  | Topology | 2-3 tier Fat Tree | 3-4 tier Fat Tree | 4+ tier Fat Tree, Megafly |
| Interoperability | Mode | Homogeneous (OPA) | Homogeneous or Heterogeneous | Heterogeneous (multi-vendor UE) |
|  | CN5000/CN6000 | ✓ OPA interop (no OPA100) | ✓ OPA interop (no OPA100) | ✓ OPA interop (no OPA100) |
| Value-Add: Performance | Link Behavior | Lossless (CBFC, LLR) | Lossless (CBFC, LLR) | Lossless (CBFC, LLR) |
|  | Message Rate | 4B msgs/sec bidirectional | 4B msgs/sec bidirectional | 4B msgs/sec bidirectional |
|  | Latency (hop) | ~110ns switch hop | ~110ns switch hop | ~110ns switch hop |
| Value-Add: Routing | SDR/Packet Spray | ✓ | ✓ | ✓ |
|  | FGAR | ✓ (telemetry-based) | ✓ (telemetry-based) | ✓ (telemetry-based) |
|  | ECAR | ✓ | ✓ | ✓ |
|  | Fast Route Recovery | ✓ (<3μs congestion response) | ✓ (<3μs congestion response) | ✓ (<3μs congestion response) |
| Value-Add: Congestion Mgmt | ECN | ✓ (for Ethernet/UE paths) | ✓ (for Ethernet/UE paths) | ✓ (for Ethernet/UE paths) |
|  | NSCC | ✓ (UE mode) | ✓ (UE mode) | ✓ (UE mode) |
|  | RCCC | ✓ (UE mode) | ✓ (UE mode) | ✓ (UE mode) |
|  | CSIG/CSIG+ | ✓ (UE mode, 2B/4B/6B) | ✓ (UE mode, 2B/4B/6B) | ✓ (UE mode, 2B/4B/6B) |
|  | OPA Congestion | ✓ (FECN/BECN native) | ✓ (FECN/BECN native) | ✓ (FECN/BECN native) |
| Value-Add: Collective Accel | In-Network | ✓ (RISC-V reduce) | ✓ (RISC-V reduce) | ✓ (RISC-V reduce) |
| Value-Add: Advanced Topo | Megafly | Optional | ✓ Supported | ✓ Optimized |
|  | Dragonfly | ✓ Supported | ✓ Supported | ✓ Supported |

## SCALE-UP SOLUTIONS

### Table 3: Scale-Up AI Workload Solutions

| Feature Category | Protocol | CN7000 Pod/Rack Scale | CN7000 Multi-Rack Scale |
| --- | --- | --- | --- |
| Protocols | Primary | UALink 200G | UALink 200G |
|  | Secondary | UE (ULN variant) | UE (ULN variant) |
|  | Fallback | — | — |
| Transports (UALink) | Semantics | Memory (load/store) | Memory (load/store) |
|  | DL/TL | Fixed 680B payload, VC, LLR, CBFC | Fixed 680B payload, VC, LLR, CBFC |
|  | Ordering | Same-address ordering | Same-address ordering |
| Transports (UE/ULN) | PDS Modes | ROD (ordered), RUD (unordered) | ROD (ordered), RUD (unordered) |
|  | CMS | RCCC (primary for incast) | RCCC (primary for incast) |
| Scale | Max Endpoints | <1K accelerators | <1K accelerators |
|  | Topology | Single-tier (1-hop) | Single-tier (1-4 racks) |
|  | Cable Length | <4m per link | <4m per link |
|  | RTT Target | <1μs | <1μs |
| Interoperability | Mode | Heterogeneous (multi-vendor XPUs) | Heterogeneous (multi-vendor XPUs) |
|  | Standards | UALink Consortium spec | UALink Consortium spec |
| Value-Add: Performance | Link Behavior | Lossless (CBFC or PFC) | Lossless (CBFC or PFC) |
|  | Message Rate | Line-rate fixed payload | Line-rate fixed payload |
|  | Latency (RTT) | Sub-μs target | Sub-μs target |
|  | Bandwidth/XPU | 1.6T (8x200G lanes) | 1.6T (8x200G lanes) |
| Value-Add: Routing | SDR/Packet Spray | N/A (single-tier, no spray) | N/A (single-tier, no spray) |
|  | FGAR | N/A (single hop) | N/A (single hop) |
|  | ECAR | N/A (single hop) | N/A (single hop) |
|  | Fast Route Recovery | N/A (static connections) | N/A (static connections) |
| Value-Add: Congestion Mgmt | ECN | Not required (lossless) | Not required (lossless) |
|  | NSCC | Not used | Not used |
|  | RCCC | ✓ (UE/ULN mode only) | ✓ (UE/ULN mode only) |
|  | CSIG/CSIG+ | Not required (single tier) | Not required (single tier) |
|  | Flow Control | PFC or CBFC mandatory | PFC or CBFC mandatory |
| Value-Add: Collective Accel | In-Network | Optional (for XPU offload) | Optional (for XPU offload) |
| Value-Add: Advanced Topo | Megafly | N/A | N/A |
|  | Dragonfly | N/A | N/A |

### Table 4: Scale-Up HPC Workload Solutions

| Feature Category | Protocol | CN7000 Pod/Rack Scale | CN7000 Multi-Rack Scale |
| --- | --- | --- | --- |
| Protocols | Primary | UE (ULN - Ultra Local Network) | UE (ULN - Ultra Local Network) |
|  | Secondary | OPA (if homogeneous Cornelis) | OPA (if homogeneous Cornelis) |
|  | Memory Semantic | Optional UALink for specific XPUs | Optional UALink for specific XPUs |
| Transports (UE/ULN) | PDS Modes | ROD, RUD | ROD, RUD |
|  | SES | RMA Write/Read, Tagged Send | RMA Write/Read, Tagged Send |
|  | CMS | RCCC (incast), NSCC (optional) | RCCC (incast), NSCC (optional) |
| Scale | Max Endpoints | <1K compute nodes | <1K compute nodes |
|  | Topology | Single-tier, all-to-all | Single-tier, 1-4 racks |
|  | Cable Length | <4m | <4m |
|  | RTT Target | <1μs | <1μs |
| Interoperability | Mode | Homogeneous or Heterogeneous | Homogeneous or Heterogeneous |
|  | Standards | UE HPC profile | UE HPC profile |
| Value-Add: Performance | Link Behavior | Lossless (CBFC, LLR) | Lossless (CBFC, LLR) |
|  | Message Rate | 4B msgs/sec | 4B msgs/sec |
|  | Latency (RTT) | Sub-μs target | Sub-μs target |
|  | Bandwidth/Node | 1.6T (aggregated) | 1.6T (aggregated) |
| Value-Add: Routing | SDR/Packet Spray | Minimal (single tier) | Minimal (single tier) |
|  | FGAR | N/A | N/A |
|  | ECAR | N/A | N/A |
|  | Fast Route Recovery | N/A | N/A |
| Value-Add: Congestion Mgmt | ECN | ✓ (available) | ✓ (available) |
|  | NSCC | Optional | Optional |
|  | RCCC | ✓ (primary) | ✓ (primary) |
|  | CSIG/CSIG+ | Optional | Optional |
| Value-Add: Collective Accel | In-Network | ✓ (MPI collectives) | ✓ (MPI collectives) |
| Value-Add: Advanced Topo | Megafly | N/A | N/A |
|  | Dragonfly | N/A | N/A |

## Feature Coexistence Rules

### Scale-Out Mandatory Combinations:

- UE+ Protocol → Requires: CBFC + LLR + (NSCC or RCCC) + CSIG capability

- Hyperscalar Deployments → Requires: FGAR + CSIG+ + (Megafly or Dragonfly topology support)

- Heterogeneous Interop → Requires: Standard UE (not UE+) + UE-compliant congestion control

- Multi-protocol Dynamic Selection → Requires: Homogeneous Cornelis deployment + OPA+UE+ capability

### Scale-Up Mandatory Combinations:

- UALink → Requires: CBFC (or PFC) + LLR + Single-tier topology + Memory semantics

- UE/ULN Scale-Up → Requires: RCCC + Sub-μs RTT + CBFC + Simplified routing

- Multi-vendor Scale-Up → Requires: UALink or UE standard compliance

### Mutually Exclusive Configurations:

- Scale-Out protocols (OPA/UE+/UE/Ethernet) ⊕ Scale-Up protocols (UALink) on same switch instance

- Dynamic protocol selection ⊕ Heterogeneous multi-vendor (dynamic selection only for homogeneous Cornelis)

- NSCC ⊕ Lossless-only mode (NSCC designed for lossy; use RCCC for lossless)

## Key Differentiators by Solution Type

| Dimension | Scale-Out AI | Scale-Out HPC | Scale-Up AI | Scale-Up HPC |
| --- | --- | --- | --- | --- |
| Primary Goal | Maximize GPU utilization, collective performance | Balanced latency/bandwidth/message rate | Ultra-low latency, memory semantics | Low-latency RDMA within pod |
| Packet Size | Variable MTU | Variable MTU | Fixed 680B (UALink) | Variable MTU |
| Routing Complexity | High (multi-tier, adaptive) | High (multi-tier, adaptive) | None (single-tier) | Minimal (single-tier) |
| Congestion Strategy | NSCC + RCCC complementary | FECN/BECN (OPA), NSCC+RCCC (UE) | RCCC only | RCCC primary |
| Telemetry | CSIG+ critical for hyperscale | CSIG+ for UE, native OPA telemetry | Not needed | Optional |
| Collective Offload | Critical (30-50% training time) | Important (MPI collectives) | Optional | Important |

References:

- CN7000 Switch Requirements (Landing Zone)

- CN7000 HFI Requirements (Landing Zone)

- UEC Transport Specifications (PDS, SES, CMS)

- UALink Consortium Specifications

- CN7000 Features Documentation